# NAS-GEM-80 FDC CARD

Instruction Manual & Functional Description

- Contents

NAS-GEM-80 FDC CARD	1

1	Introduction	3

2	Components	4

3	Construction	6

3.1	Before you start construction	6

3.2	Order of construction	6

4	Functionality	7

4.1	Links	7

4.2	SW1 – Base Port Select	7

4.3	LKB2 – Configuration Switch	8

4.4	Ports	8

4.4.1	For port 0xE4	8

4.4.2	For port 0xE5	9

5	Configuration	10

5.1	Without A Scope	10

5.2	With A Scope	10

6	Notes on Components	11

6.1	LKB2 Configuration Switch	11

6.2	SW1 Decode Header	11

6.3	C4	11

6.4	IC’s U15, U20, U21	11

6.5	1793 FDC Controller	11

7	Errata	16

8	Reference Images	17

# Introduction

This card has been designed to be a form, fit and function replacement for the Nascom Floppy Disk Controller, which is proving to be a very difficult item to source for the Nascom / Gemini retro community.

The card has been tested against original Teac FD-50E/F drives without observed issues under NAS-DOS 1.4

Further testing with CP/M and PolyDos is planned

Note: The card is designed to handle 5.25-inch drives. Support for 8-inch drives has been removed (Did anyone ever use this?)

# Components

| Reference | Quantity | Value |
| --- | --- | --- |
| Capacitors |  |  |
| C1 (Tantalum) | 1 | 1uF |
| C2 | 1 | 33pF |
| C3 | 1 | 150pF |
| C4 (Polyester Film) | 1 | 4n7 |
| C5 | 1 | 1nF |
| C6 (Tantalum) | 1 | 100uF |
| C7 (Tantalum) | 1 | 22uF |
| C8, C9, C101 - C106 (Tantalum) | 8 | 10uF |
| DC1 - DC24 – Decoupling caps | 25 | 10nF |
| Resistors | Quantity | Value |
| R1 | 1 | 100K |
| R2 | 1 | 5K6 |
| R3 | 1 | 4K7 |
| R4 | 1 | 10K |
| R5 | 1 | 22K |
| R6 | 1 | 1M |
| R7 | 1 | 1M |
| R8 | 1 | 15K |
| R9 | 1 | 1K |
| R10 | 1 | 4K7 |
| R11 | 1 | 150 |
| R12 | 1 | 150 |
| R13 | 1 | 150 |
| R14 | 1 | 150 |
| R15 | 1 | 150 |
| R16 | 1 | 10K |
| R17 | 1 | 10K |
| R18 | 1 | 10K |
| R19 | 1 | 10K |
| R20 | 1 | 10K |
| R21 | 1 | 10k |
| R22 | 1 | 10K |
| R23 | 1 | 10K |
| R24 | 1 | 10K |
| R25 | 1 | 10K |
| R26 | 1 | 4K7 |
| R27 | 1 | 4K7 |
| R28 | 1 | 220K |
| R29 | 1 | 150K |
| R30 | 1 | 1K0 |
| R31 | 1 | 470 |
| IC’s | Quantity | Value |
| U1 – DIP 20 | 1 | 74LS245 |
| U2 – DIP 16 | 1 | 74LS138 |
| U3 – DIP 14 | 1 | 74LS10 |
| U4 – DIP 14 | 1 | 74LS04 |
| U5 – DIP 16 | 1 | 74LS365 |
| U6 – DIP 16 | 1 | 74LS163 |
| U7 – DIP 16 | 1 | 74LS257 |
| U8 – DIP 16 | 1 | 74LS257 |
| U9 – DIP 14 | 1 | 74LS32 |
| U10 – DIP 14 | 1 | 74LS32 |
| U11 – DIP 14 | 1 | 74LS02 |
| U12 – DIP 16 | 1 | 74LS195 |
| U13 – DIP 20 | 1 | 74LS273 |
| U14 – DIP 40 | 1 | WD1793 |
| U15 – DIP 14 (Static sensitive) | 1 | 4013B |
| U16 – DIP 14 | 1 | 7407 |
| U17 – DIP 14 | 1 | 74LS04 |
| U18 – DIP 16 | 1 | 74LS123 |
| U19 – DIP 14 | 1 | 7438 |
| U20 – DIP 16 (Static sensitive) | 1 | 4046B |
| U21 – DIP 14 (Static sensitive) | 1 | 4016 |
| U22 – DIP 14 | 1 | 74LS14 |
| U23 – DIP 14 | 1 | 7406 |
| U24 – DIP 16 | 1 | 74LS123 |
| Other Items | Quantity | Value |
| D1 – LED | 1 | LED 3mm |
| LK1, LK3, LK4 (optional) | 3 | Switch SPST Slide, 6.7x4.1mm, W7.62mm, P2.54mm |
| LK2 | 1 | Switch DPDT, 3 pins, P2.54mm |
| RV1 | 1 | 50K, Cermet type potentiometer |
| SW1 | 1 | Switch or header, x8, W7.62mm_Socket |
| LKB2 | 1 | Switch SW_DIP_x10, W7.62mm_Socket |
| PL1 | 1 | Connector, Header 2x17, P2.54mm |

# Construction

## Before you start construction

Inspect the PCB for any visible signs of damage

Plug the board into an 80-BUS and power it up.

- Verify no latent shorts exist

- Verify correct voltages on power rails

Select your components:

- Turned pin sockets are recommended due to robustness and reliability

- Tantalum capacitors can be temperamental. Make sure they are inserted with the correct polarity, are of good quality and are overrated voltage wise.

U15, U20 & U21 are static sensitive. Handling precautions need to be observed.

## Order of construction

The recommended order of construction is:

- Resistors

- Sockets

- Decoupling capacitors

- Tantalum capacitors

- Switches

- LED

- Wire header

- Insert IC’s

# Functionality

## Links

The board contains 4 links, which can be implemented as switches or hard wired with links depending on the user’s preference.

| Link (Default) | Usage |
| --- | --- |
| LK1 Open | Ready |
| LK2 A -> C | Side Select. Set A -> C for 1793 Side Select |
| LK3 Open | Spare – Port x4 Bit 5 |
| LK4 Open | Spate – Port x4 Bit 7 |

## SW1 – Base Port Select

This allows the board to be mapped to ports 0x20, 0x40, 0x60, 0x80, 0xA0, 0xC0, 0xE0. The default is 0xE0.  User options are to either hard wire a link, use a link block or use an 8-way DIL switch.

| Connect Pins | Port Base Address |
| --- | --- |
| 1 to 16 | N/A |
| 2 to 15 | 0x20 |
| 3 to 14 | 0x40 |
| 4 to 13 | 0x60 |
| 5 to 12 | 0x80 |
| 6 to 11 | 0xA0 |
| 7 to 10 | 0xC0 |
| 8 to 9 (Default) | 0xE0 |

## LKB2 – Configuration Switch

This is implemented as a 10-way DIL switch. The switch settings are as follows:

| Switch | Usage | 2MHz Default | 4MHz Default |
| --- | --- | --- | --- |
| 1 | Set write pre-compensation | Off | Off |
| 2 | Only one of SW1, SW2 or SW3 should be selected | Off | On |
| 3 | … | Off | Off |
| 4 | Clock divide by 2 (For 2MHz systems) | On | Off |
| 5 | Clock divide by 4 (For 4MHz systems) | Off | On |
| 6 | Clock divide by 4 (For 2MHz systems), VCO=500KHz | On | Off |
| 7 | Clock divide by 8 (For 4MHz systems), VCO=500KHz | Off | On |
| 8 | Select single /. double density | On | On |
| 9 | Disable / enable track 43 pre-compensation | On | On |
| 0 | Disable write pre-compensation | On | Off |

## Ports

The board uses six ports, selected from the base port (Default 0xE0).  These are:

| Port | Read | Write |
| --- | --- | --- |
| 0xE0 | 1793 Status register | 1793 Command register |
| 0xE1 | 1793 Track register | 1793 Track register |
| 0xE2 | 1793 Sector register | 1793 Sector Register |
| 0xE3 | 1793 Data register | 1793 Data register |
| 0xE4 | Drive selection | Drive select control |
| 0xE5 | INTRQ / DRQ status | N/A |

### For port 0xE4

| Bit | Read | Write |
| --- | --- | --- |
| 0 | Drive select 0 | Drive select 0 |
| 1 | Drive select 1 | Drive select 1 |
| 2 | Drive select 2 | Drive select 2 |
| 3 | Drive select 3 | Drive select 3 |
| 4 | Side select | Side select (When LK2 A->C) |
| 5 | LK3 | Motor 0=off, 1=on |
| 6 | Density (0=low, 1 = high) | Density (0=low, 1 = high) |
| 7 | LK4 | N/A |

### For port 0xE5

| Bit | Read | Write |
| --- | --- | --- |
| 0 | 1793 INTRQ | N/A |
| 1 | 0=Ready, 1=Not Ready | N/A |
| 2 | N/A | N/A |
| 3 | N/A | N/A |
| 4 | N/A | N/A |
| 5 | N/A | N/A |
| 6 | N/A | N/A |
| 7 | 1793 DRQ | N/A |

# Configuration

Once the board has been built and configured via links options and switches, the VCO center frequency needs to be set.  This can be done in two ways:

## Without A Scope

The following steps are required to perform the configuration:

- Ensure the board is disconnected from any drives (Disconnect PL1)

- Turn the potentiometer at VR1 anti-clockwise until the LED illuminates

- Turn the potentiometer at VR1 a quarter turn clockwise

## With A Scope

The following steps are required to perform the configuration:

- Ensure the board is disconnected from any drives (Disconnect PL1)

- Observe the signal at U20 Pin 3

- Turn the potentiometer until a 500KHz square wave is observed

# Notes on Components

All the components used have been selected at time of design to be readily available via eBay and other sources.

## LKB2 Configuration Switch

It is recommended to fit a 20-pin socket and use a 10-way DIL switch

## SW1 Decode Header

The socket at SW1 is recommended to be of the turned pin variety so as to allow easier insertion of the header. The default header is a 16-way DIL header.

An alternative is to use Single-In-Line (SIL) Turned Pin Socket 0.1 Inch Pitch strips which are readily available instead of a header.  These are then snipped down to the desired length before being inserted into the socket and the desired wire links being made.

An 8-way DIL switch may also be used if one is available.

## C4

This capacitor is of the film type in the original design.  I have used an Axial Polypropylene Film Capacitor here without issue. Other types (e.g. ceramic) may work but that configuration is untested.

## IC’s U15, U20, U21

These parts are static sensitive. Handling precautions need to be observed.

## 1793 FDC Controller

A 1797 controller, in theory, may be substituted, but that configuration is untested. To use the 1797 Side-Select feature, set LK3 to B -> C

# Errata

Initial release boards (1.2.0) have an incorrect ground line for C1.

The capacitor at location C1 should be:

- Mounted on the back of the board

- The positive leg should go through the existing positive through plated hole for the component

- The negative leg should go to pin 14 of U24

This modification fixes an issue with the unstable operation of the Head Load monostable

# Reference Images

<Insert photo’s here>